CONTENTS

2.	PRE-PROGRAMMED DATABASE	1

2.1	COUNTRY CODE ASSIGNMENT  (PGM 100)	1

2.2	RACK SLOT ASSIGNMENT  (PGM 101)	4

2.3	WTIB PORT NUMBER ASSIGN  (PGM 102)	5

2.4	LOGICAL SLOT ASSIGNMENT  (PGM 103)	6

2.5	NUMBERING PLAN TYPE  (PGM 104)	7

2.6	FLEXIBLE NUMBERING PLAN  (PGM 105)	8

2.7	FLEXIBLE NUMBERING PLAN  (PGM 106 - 107)	9

2.8	IP SETTING  (PGM 108)	12

# 2.	PRE-PROGRAMMED DATABASE

This section describes the pre-programming of the data. When installed, user must program some database (ex. Nationality, Numbering Plan Type, and Slot Assignment) first, otherwise system may not operate properly.

If you want to change the nation, then use PGM 100. To assign the board in each rack, proceed PGM 101-102. After rack slot assignment, you must adjust PGM 103, logical slot assignment program. If you want to use the station number range flexibly, you can select a proper numbering plan type at PGM 104. If you want to change station numbers or feature codes, then use PGM 105~107. If you need more than 4 digits station number for VPN or QSiG, use the station prefix code at PGM 100.

PROCEDURE FOR PRE-PROGRAMMING

- Set Nationality (See Section 2.1)

- Set DIP switch 8 as ‘OFF’, and press reset button. Then slot assignment is configured.

- Set DIP switch 8 as ‘ON’ to retain current configuration.

- Set Rack Slot Assignments (If configuration is not equal to the system default slot assignments, you must program it manually. See Section 2.2)

- Set Logical Slot Assignment. (After this configuration or the rack slot assignment, system must be reset manually. See Section 2.4)

- Set Numbering Plan Type. (See Section 2.5)

- Set Flexible Numbering Plan. (See Section 2.6)

- Set Station Prefix Code. (Only for VPN or QSiG. See Section 2.1)

## 2.1	COUNTRY CODE ASSIGNMENT  (PGM 100)

PROCEDURE

| LOCATION PROGRAM PRESS FLEX_KEY (1-2) | - [TRANS/PGM] + 100. |
| --- | --- |
| NATION CODE 82 | - User can select one flex BTN of the three BTNs. (EX. In case 1st flex BTN) LCD shows default or saved value of NATION CODE. Enter desired nation code in the TABLE 2.1.2. To save nation code in memory, press [HOLD/SAVE] button. |
| LOCATION PROGRAM PRESS FLEX_KEY (1-2) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Nation Code |  | 82 | Max 2 digits |
| 2 | Site Name |  | .. | Max 23 digit (Refer to English Character Set) |

TABLE 2.1.1	 Location Program (PGM 100)

| NATION | CODE | NATION | CODE | NATION | CODE |
| --- | --- | --- | --- | --- | --- |
| America | 1 | Argentina | 54 | Australia | 61 |
| Bahrain | 973 | Bangladesh | 880 | Belgium | 32 |
| Bolivia | 591 | Brazil | 55 | Brunei | 673 |
| Burma | 95 | Cameroon | 237 | Chile | 56 |
| China (Taiwan) | 886 | CIS | 7 | Colombia | 57 |
| Costa Rica | 506 | Cyprus | 357 | Czech | 42 |
| Denmark | 45 | Ecuador | 593 | Egypt | 20 |
| El Salvador | 503 | Ethiopia | 251 | Fiji | 679 |
| Finland | 358 | France | 33 | Gabon | 241 |
| Germany | 49 | Ghana | 233 | Greece | 30 |
| Guam | 671 | Guatemala | 502 | Guyana | 592 |
| Haiti | 509 | Honduras | 504 | Hongkong | 852 |
| India | 91 | Indonesia | 62 | Iran | 98 |
| Iraq | 964 | Ireland | 353 | Israel | 972 |
| Italy | 39 | Japan | 81 | Jordan | 962 |
| Kenya | 254 | Korea | 82 | Kuwait | 965 |
| Liberia | 231 | Libya | 218 |  |  |
| Luxembourg | 352 | Malaysia | 60 | Malta | 356 |
| Mexico | 52 | Monaco | 377 | Morocco | 212 |
| Netherlands | 31 | New Zealand | 64 | Nigeria | 234 |
| Norway | 47 | Oman | 968 | Pakistan | 92 |
| Panama | 507 | P.N.G | 675 | Paraguay | 595 |
| Peru | 51 | Philippines | 63 | Portugal | 351 |
| Qutar | 974 | Saudi Arabia | 966 | Senegal | 221 |
| Singapore | 65 | South Africa | 27 | Spain | 34 |
| Sri Lanka | 94 | Swaziland | 268 | Sweden | 46 |
| Switzerland | 41 | Thailand | 66 | Tunisia | 216 |
| Turkey | 90 | U.A.E. | 971 | United Kingdom | 44 |
| Uruguay | 598 | Venezuela | 58 | Y.A.R. | 967 |

TABLE 2.1.2	Nation & National Code  (PGM 100)

| Q - 11 Z - 12 . - 13 1 - 10 | A - 21 B - 22 C - 23 2 - 20 | D - 31 E - 32 F - 33 3 - 30 |
| --- | --- | --- |
| G - 41 H - 42 I - 43 4 - 40 | J - 51 K - 52 L - 53 5 - 50 | M - 61 N - 62 O - 63 6 - 60 |
| P - 71 R - 72 S - 73 Q - 7* 7 - 70 | T - 81 U - 82 V - 83 8 - 80 | W - 91 X - 92 Y - 93 Z - 9# 9 - 90 |
| *1 - Blank *2 - : *3 - , | 0-00 | # |

TABLE 2.1.3  English Character Set

## 2.2	RACK SLOT ASSIGNMENT  (PGM 101)

Rack slot assignment can be performed automatically and manually. If DIP switch of the manual board detection is OFF, system will detect the installed board type automatically. If DIP switch is ON, the board type code must be entered at each slot. After manual RACK slot assignment, user must reset the system manually.

PROCEDURE

| BOARD ASSIGNMENT ENTER SLOT NUMBER | - [TRANS/PGM] + 101 |
| --- | --- |
| SLOT 01 (F1:ID F2:DEVS) ID : DTIB12 DEVS: 12 | - To program the slot assignment, Dial two digits(01-27). Two digits entered indicates a slot number. If indicated slot is pre-slot assigned, related message will be displayed on LCD. |
| SLOT 01 (F1:ID F2:DEVS) ID : SLIB6 DEVS: 6 | - To change pre-assigned board, Press Flex Btn 1 and dial 2 digits related board type code. Table 2.2.1 illustrates each board type code. Then entered board type code related board message will be displayed on LCD. |
| SLOT 01 (F1:ID F2:DEVS) ID : PRIB DEVS: 30 | c.f) In case of PRIB, logical port assign is possible. To program a logical port number, after entering board type code, press Btn 2and dial 2 digits logical port number between 01-maximum board port capacity. If logical port doesn’t enter, maximum port will be assigned. |
| SLOT 01 (F1:ID F2:DEVS) ID : PRIB DEVS: 30 | - Press the [HOLD/SAVE] button for saving database permanently. |
| BOARD ASSIGNMENT ENTER SLOT NUMBER | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| STA | CODE | COL | CODE | STA & COL | CODE | Etc | CODE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DTIB12 DTIB24 SLIB6 SLIB12 WTIB DSIB | 11 12 13 14 15 18 | PRIB BRIB LCOB4 LCOB8 TLIB EMIB | 31 32 33 34 37 38 | STIB | 51 | VMIB | 61 |

TABLE 2.2.1   Board Type Code Table  (PGM 101)

## 2.3	WTIB PORT NUMBER ASSIGN  (PGM 102)

This procedure determines the number of registrable wireless terminals. The available range is from 008 to 192, and default value is 008.

PROCEDURE

| WTIB PORT NO (008–192) 008 (MULTIPLE OF 8) | - [TRANS/PGM] + 102. |
| --- | --- |
|  | - Press number of registrable wireless terminals. The number should be within the range between 008 to 192, and be the multiple of 8. |
| WTIB PORT NO (008–192) 032 (MULTIPLE OF 8) | - Press the [HOLD/SAVE] button for updating database permanently. |

## 2.4	LOGICAL SLOT ASSIGNMENT  (PGM 103)

Logical slot assignment can be performed automatically and manually. If DIP switch of the manual board detection is OFF, system will detect the logical slot assign sequence as increase order automatically. If DIP switch is ON, the logical slot assignment must be entered at each board type. After manual logical slot assignment, user must reset the system manually. Logical slot assignment of VMIB doesn’t perform automatically. So user must assign VMIB slot number manually. In case of WTIB one WTIB board is detected by logical slot number because WTIBs operate like one board

PROCEDURE

| LOGICAL SLOT ASSIGN COL STA VMIB | - [TRANS/PGM] + 103 |
| --- | --- |
| LOGICAL SLOT ASSIGN F1 : 1-16 F2 : 17-27 | - To program the logical board assign sequence of the COL, STA, and VMIB, press the related flexible button. Press Flex BTN1 or BTN2 related with slot number assigned. |
| 02 03 .. .. .. .. .. .. .. .. .. .. .. .. .. .. | - Press BTN1 and dial slot numbers with increasing order for logical slot numbers. If slot numbers is more than 16, Press [HOLD], [CONF] and BTN2 and then dial other slot number. |
| 02 03 05 07 .. .. .. .. .. .. .. .. .. .. .. .. | - Press the [HOLD/SAVE] button for saving database permanently. |
| LOGICAL SLOT ASSIGN F1 : 1-16 F2 : 17-27 | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- |
| 1 | COL Board | - | DIP ON: Manually DIP OFF: Automatically |
| 2 | STA Board | - | DIP ON: Manually DIP OFF: Automatically |
| 3 | VMIB | - | Only programmed manually |

TABLE 2.4.1		Button Configuration for Slot Assignment  (PGM 103)

## 2.5	NUMBERING PLAN TYPE  (PGM 104)

PROCEDURE

| NUMBERING PLAN (1-8) 1 | - [TRANS/PGM] + 104. |
| --- | --- |
| NUMBERING PLAN (1-8) 2 | - To program the numbering plan type, press one digit between 1-8. |
| NUMBERING PLAN (1-8) 2 | - Press the [HOLD/SAVE] button for saving database permanently. |

| BTN | ITEM | INTERCOM RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Number Set Type 1 | 100 – 399 | Yes | As the basic type, the 1st digit of station number should be 1 – 4. |
| 2 | Number Set Type 2 | 100 – 399 | No | The station number can be changed within 799. |
| 3 | Number Set Type 3 | 100 – 399 | No | Australia Default |
| 4 | Number Set Type 4 | 7700-7999 | No | New Zealand Default |
| 5 | Number Set Type 5 | 200 – 499 | No | Italy Default |
| 6 | Number Set Type 6 | 21 – 79 | No | Finland Default Max Station Ports 60 |
| 7 | Number Set Type 7 | 10 – 79 | No | Max Station Ports 70 |
| 8 | Number Set Type 8 | 100 – 399 | No | The station number can be changed within 999. |

TABLE 2.5.1		Flexible Numbering Plan  (PGM 104)

## 2.6	FLEXIBLE NUMBERING PLAN  (PGM 105)

Station Number of the system can be assigned flexibly via the system programming. Station Numbering Code length should be in the range of 1(one) digit through 4 digits.

PROCEDURE

| 000 001 002 003 100 101 102 103 | - [TRANS/PGM] + 105. |
| --- | --- |
| 000 001 002 003 100 101 102 103 | - Station Numbers Assign. you will see the 4 station numbers corresponding to the 4 port numbers. Station number length is in the range of 2 digit through 4 digits. There are two methods for changing station number. - Dial two station numbers - Range start station number & range end station number, then LCD shows dialed range value. Press the [HOLD/SAVE] button, then station numbers changed from the first station number on current LCD to range end (All LEDs of BTN are off.). - Press one of BTN 1-4 (Each BTN 1-4 is assigned to station number 1- 4 on the current LCD), then LED of pressed Flex button is steady on. Dial new station number and press the [HOLD/SAVE] button, or press other Flex button to assign stn number to other station without saving (The LED of pressed Flex. button is on.). - If you want to delete all station numbers, press the [SPEED] button and press [HOLD/SAVE] button, then all station numbers are cleared. - If you want to change next 4 station numbers then press [] button. If you want to change previous 4 station numbers, then press [] button. |
| 000 001 002 003 100 400 102 103 | - Press the [HOLD/SAVE] button for saving database permanently. (ex: Press Flex btn2, dial 400 and press [HOLD/SAVE] ) |

## 2.7	FLEXIBLE NUMBERING PLAN  (PGM 106 - 107)

Feature codes of the system can be assigned flexibly via the system programming. Feature Code length should be in the range of 1(one) digit through 4 digits. Let's say that a feature code conflict has occurred in such a case that a feature code string matches with other longer feature code string, checking from the first digit of the code. For example, features Code 53 and 536 have a feature code conflict. The system will not allow any feature code conflict.

PROCEDURE

| FLEX NUMBERING PLAN A PRESS FLEX KEY (01-24) | - [TRANS/PGM] + 106. You can program the 24 Flex. Numbers. - Table 2.7.1 illustrates the programmable list of the 24 flexible - Numbering plan used by code 106 and Table 2.7.2 illustrates the - List for code 107. |
| --- | --- |
| STA GRP PILOT NUMBER START & END #(620-667) | - To change the other numbering plan, press the related flexible button. This illustrates the station group pilot number. Change with entering range (ex. Pilot Group Assign). Each code length is in 1~4 digits. Dial the first Station Group Pilot Number and Last Station Group Pilot Number, and press the [HOLD/SAVE] button. Then you can see the changed Station Group Pilot Numbers on the LCD. If you press other Flex button, go to the other Flexible number assign . |
| STA GRP PILOT NUMBER START & END #(620-667) | - If you want to save all changed Flexible numbers to system memory, press the [HOLD/SAVE] button. There are no errors in Flexible Numbers, then confirmation tone will be received. |
|  | - But errors are detected, then error tone will be received without updating system memory. |

| BTN | LCD DISPLAY | ITEM |
| --- | --- | --- |
| 1 | STA GRP PILOT NUMBER START & END #(620-667) | Station Group Pilot Number |
| 2 | INT PAGE ZONES START & END #(501-530) | Internal Page Zone Number |
| 3 | INT ALL CALL ENTER NEW #(543) | Internal All Call Page |
| 4 | MEET ME PAGE ENTER NEW #(544) | Meet Me Page |
| 5 | EXT PAGE ZONE 1 ENTER NEW #(545) | External Page Zone - 1 |
| 6 | EXT PAGE ZONE 2 ENTER NEW #(546) | External Page Zone - 2 |
| 7 | EXT PAGE ZONE 3 ENTER NEW #(547) | External Page Zone - 3 |
| 8 | EXT ALL CALL ENTER NEW #(548) | External All Call Page |
| 9 | ALL CALL PAGE ENTER NEW #(549) | All Call Page (Internal/External) |
| 10 | SMDR ACT CODE ENTER ENTER NEW #(550) | SMDR Account Code Enter |
| 11 | FLASH CMD TO CO ENTER NEW #(551) | Flash Command to CO Line |
| 12 | SLT LAST SPD DIAL ENTER NEW #(552) | Last Number Redial (LNR) |
| 13 | DND ENTER NEW #(553) | Do-Not-Disturb |
| 14 | CALL FWD ENTER NEW #(554) | Call Forward |
| 15 | SPD DIAL PGM ENTER NEW #(555) | Speed Dial Program |
| 16 | MSG WAIT ENABLE ENTER NEW #(556) | MSG Wait/Call-Back Enable |
| 17 | MSG WAIT RETURN ENTER NEW #(557) | MSG Wait/Call-Back Answer |
| 18 | SPD DIAL ACCESS ENTER NEW #(558) | Speed Dial Access |
| 19 | DND/FWD CANCEL ENTER NEW #(559) | Cancel DND/CFW/Pre-selected MSG Features |
| 20 | SYSTEM HOLD ENTER NEW #(560) | System hold |
| 21 | STA RELOC BACKUP ENTER NEW #(561) | Backup STA relocation |
| 22 | STA RELOC RETRIEVE ENTER NEW #(562) | Retrieve STA relocation |
| 23 | SLT PGM MODE ENTER ENTER NEW #(563) | SLT Program Mode Select |
| 24 | ALARM RESET ENTER NEW #(565) | Alarm Reset |

TABLE 2.7.1 The Flex Numbering PLAN A Configuration (PGM 106)

| BTN | LCD DISPLAY | ITEM |
| --- | --- | --- |
| 1 | GROUP CALL PKUP ENTER NEW #(566) | Group Call Pick-Up |
| 2 | UCD DND ENTER NEW #(568) | Ucd Group DND |
| 3 | NIGHT ANSWER ENTER NEW #(569) | Night Answer |
| 4 | CALL PARK LOCATIONS START & END #(601-619) | Call Park Locations |
| 5 | DIRECT CALL PKUP ENTER NEW #(7) | Direct Call Pick-Up |
| 6 | ACCESS CO GROUP FEAT START & END #(801-872) | Access CO Group |
| 7 | ACCESS IND CO FEAT START&END #(88001-88200) | Access Individual CO Line |
| 8 | TIE ROUTING ACCESS ENTER NEW #(8901) | TIE Routing Access |
| 9 | ACCESS HELD CO FEAT ENTER NEW #(8*) | Access Held CO Group |
| 10 | ACCESS HELD IND CO FEAT START&END # (8#001-8#370) | Access Held Individual CO Line |
| 11 | ACCESS CO IN 1ST CO GRP ENTER NEW #(9) | Access to CO line in the 1st available CO Group |
| 12 | ATTENDANT CALL ENTER NEW #(0) | Attendant Call |
| 13 | DOOR OPEN 1 ENTER NEW #(#*1) | Door Open – 1 |
| 14 | DOOR OPEN 2 ENTER NEW #(#*2) | Door Open – 2 |
| 15 | DOOR OPEN 3 ENTER NEW #(#*3) | Door Open – 3 |
| 16 | DOOR OPEN 4 ENTER NEW #(#*4) | Door Open – 4 |
| 17 | DOOR OPEN 5 ENTER NEW #(#*5) | Door Open – 5 |
| 18 | DOOR OPEN 6 ENTER NEW #(#*6) | Door Open – 6 |
| 19 | DOOR OPEN 7 ENTER NEW #(#*7) | Door Open – 7 |
| 20 | VM MSG WAIT ENABLE ENTER NEW #(*8) | VM MSG Wait Enable |
| 21 | VM MSG WAIT CANCEL ENTER NEW #(*9) | VM MSG Wait Cancel |

TABLE 2.7.2  The Flex Numbering PLAN B Configuration   (PGM 107)

## 2.8	IP SETTING  (PGM 108)

IP address, Subnet mask, GW address should be programmed for trace, remote upgrade, Pc_adm, Pc_atd and etc via network.

PROCEDURE

| IP NET SETTING PRESS FLEX_KEY (1-5) | - [TRANS/PGM] + 108. |
| --- | --- |
| IP NAME LGICKEYPHONE | - For IP Name assignment press Flex BTN 1, then enter the character. |
| SERVER IP ADDR(SKIP:#) 165.147. 3. 1 | - For Server Ip assign press Flex BTN 2, then enter the 12digit. - To skip entering digit, press #. |
| SERVER IP ADDR(SKIP:#) 165.147. 3. 1 | - Press the [HOLD/SAVE] button for saving database permanently. |
| IP NET SETTING PRESS FLEX_KEY (1-5) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | INTERCOM RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | IP NAME | 16 Chars | - | IP Name |
| 2 | SERVER IP ADDR | 12 Digits | - | Sever Address |
| 3 | CLI IP ADDR | 12 Digits | - | Cli Ip Address |
| 4 | GATEWAY ADDR | 12 Digits | - | Gateway Address |
| 5 | SUBNET MASK | 12 Digits | - |  |

TABLE 2.8.1		IP Setting  (PGM 108)